# DATE: 2025-10-12

# REVISION: -  A

# TO: Geoffrey Butler, AE460 Instructor

# FROM:

# Aleborn

# Bryce Sullins

# Maxwell O’Neill  MO

SUBJECT: Landing Gear Sizing of AIAA Carrier Strike Fighter Aircraft

# OBJECTIVE

Use the weight estimates to determine the loads on each strut on the main and nose landing gear. Those loads are then used to determine which tires will be chosen for use. Include a drawing of the landing gear layout with the associated loads.

# SUMMARY

Preliminary landing gear loads and location were determined using previously obtained weight estimates derived from TM-2 with formulas found in “Part IV: Layout of Landing Gear and Systems” (Roskam) with the idea of implementing a tricycle layout for the landing gear. As the design of the aircraft progresses, there will be multiple iterations and calculations that will change the landing gear loads.

Since the center of gravity for the aircraft has not been determined and finalized, a rough estimate for the loads that the nose and main struts will encounter were predicted. These loads were calculated as a percentage of the maximum ramp weight (1.01 times the max takeoff weight), with the total nose gear load being ten percent (10%) of the maximum ramp weight and the total main gear load being ninety percent (90%) of the maximum ramp weight. The aircraft will have one nose landing gear with two tires and two main landing gears with one tire each. The overall load experienced for both the nose and main landing gear tires will be divided by two to choose the best tire for each location.

# SUPPORTING DOCUMENTATION

Table 1: Tire size options and specs – Team 2 Next-Gen Carrier-Based Fighter

|  | Tire Size | Ply | Static Load Rating (lbs) | Speed Rating | Inflation Pressure | Max Tire OD | Max Width |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main | 35x11.5-17 | 26 | 28300 | 240 | 255 | 35 | 11.5 |
| * | 32x11.5-15 | 26 | 27800 | 210 | 290 | 32 | 11.5 |
|  | 30x11.5-14.5 | 24-26 | 25000-26600 | 210-220 | 243-265 | 29.75 | 11.5 |
| Nose | 23x7-12 | 12 | 7800 | 210 | 160 | 23.2 | 7.2 |
| * | 22x6.6-10 | 18-22 | 9200-14150 | 190-225 | 260-304 | 22.2 | 6.8 |
|  | 21x7.25-10 | 12-20 | 6400-12000 | 225-195 | 166-325 | 21.25 | 7.2 |
|  | 18x5.7-8 | 18-20 | 8600-9000 | 250 | 300-315 | 17.9 | 5.7 |

Tire size and information obtained from 2022 Goodyear aviation data book

Tire sizes with an asterisk next to them indicate those used on the F-18 Super Hornet, so those will be considered as the baseline with a size above and below that to offer other options depending on what parameters need to be met. Some tire sizes have multiple options to choose from so there is a range of values in different specification categories, as seen in Table 1. The final expected load and speed rating will help determine the best tire for the aircraft.

Figure 1: Landing gear layout with loads for nose and main landing gear