DATE: 2025-11-30

REVISON: A

TO: Geoffrey Butler, AE460 Instructor

FROM:

Aleborn

Ethan Barichievich

Maxwell O’Neill MO

SUBJECT: Wing and Stabilizer Layout for AIAA Carrier-Based Strike Fighter

OBJECTIVE

Using the tail volume results from TM-5 as the baseline for TM-6, a planform and side view CAD layout of the main wing, horizontal, and vertical stabilizers were constructed in SolidWorks. The memo provides the geometric parameters required for the drawing, which includes and coordinate system, planform areas, aspect ratios, taper ratios, sweep angles, mean aerodynamic chord, as well as relative fuselage location with relation to the wing and tail surfaces. A top view and side view were modeled as part of the conceptual phase.

SUMMARY

The final conceptual layout displays a 41.5 ft-span with a tapered main wing positioned approximately 250 (in) after FS0 for the leading-edge MAC. Previously, the horizontal stabilizer area was calculated to be 90 (ft2) which is located 18.08 (ft) aft of the wing quarter chord. This also gives a horizontal tail volume coefficient of 0.1634. The conceptual design has twin fin arrangements for the vertical stabilizers with a total aera of 101.77 (ft2) which is located 17.88 (ft2) aft of the wing’s quarter chord. Using these parameters the vertical tail coefficient was determined to be 0.0803. All sizing parameters trace to TM-2, TM-3, and TM-5. This geometry specified in this memo will be used for TM-7 Takeoff, Landing, and Balanced Field Length as well as TM-8 Layout and Mass Properties.

SUPPORTING DOCUMENTATION

Table 1: Wing Geometry

| Parameter | Units | Value |
| --- | --- | --- |
| Planform area | Sh (ft2) | 600 |
| Span | B (ft) | 41.5 |
| Aspect Ratio | AR | 2.87 |
| Taper ratio | λ | 0.2 |
| Sweep (LE ¼ c, ½ c, TE) | Deg | 43 |
| Root Chord | in | 289.157 |
| Tip Chord | in | 45.337 |
| MAC Length | in | 199.196 |
| Wing LE MAC from FSO | in | 340.299 |

Table 2: Horizontal Stabilizer Geometry

| Parameter | Units | Value |
| --- | --- | --- |
| Planform area | Sh (ft2) | 90 |
| Span | B (ft) | 19 |
| Aspect Ratio | AR | 4.0 |
| Taper ratio | λ | 0.40 |
| Tail Volume Coefficient | Vh | 0.1634 |
| Moment Arm (Wing 1/4c -tail1.4c) | Ih | 18.08 |
| Sweep (1/4 c line) | deg | 29 |

Table 3: Vertical Stabilizer Geometry

| Parameter | Units | Value |
| --- | --- | --- |
| Planform area | Sh (ft2) | 101.77 |
| No. of Fins | -- | 2 |
| Area Per Fin | ft2 | 50.88 |
| Span | B (ft) | 6 |
| Aspect Ratio | AR | 2.823 |
| Taper ratio | λ | 0.2 |
| Tail Volume Coefficient | Vh | 0.08025 |
| Moment Arm (Wing 1/4c -tail1.4c) | Ih | 17.875 |
| Sweep (1/4 c line) | deg | 35 |

Figure 1: Layout

REFERENCES

[1.] Roskam, J., Airplane Design, Part II: Preliminary Configuration Design and Integration of the Propulsion System, DARcorporation, 1985.

[2.] Pamadi, B. N., Performance, Stability, Dynamics, and Control of Airplanes, 3rd ed., AIAA Education Series, 2017.

[3.] Team 2 (Aleborn), TM-2 Preliminary Weight Estimates, SDSU AE 460A, 2025-09-28.

[4.] Team 2 (Aleborn), TM-3 Wing Sizing, SDSU AE 460A, 2025-10-05.

[5.] Team 2 (Aleborn), TM-5 Empennage Sizing, SDSU AE 460A, 2025-10-19.

[6.] AIAA, 2025–2026 Undergraduate Aircraft Design Competition RFP: Next-Generation Carrier-Based Strike Fighter, AIAA, 2025.